Special Note:

The BIOS files provided here are exclusively for our products. We take no responsibility for any damage caused by improper use or lack of technical expertise.

Attention:

Users who download BIOS from here should use the flash utility included in the downloaded compressed file when doing the BIOS update. To avoid BIOS update failure, please do not use older versions of the update utility or utilities not provided by us. Always use AC adapter while updating the BIOS, any shutdown during the update will cause system failure.

Requirements:

- BIOS updates need to be run from a bootable floppy disk. An external USB Floppy Drive is required.

- Or to use a bootable USB flash drive (Pen drive) to update BIOS as well.

For how to make a bootable USB flash drive, please check http://www.howtogeek.com/136987/how-to-create-a-bootable-dos-usb-drive/

You need to disable the UEFI in the BIOS setup, please check your user’s manual for how to disable the UEFI.

Procedure:

1. Extract the zip file containing the BIOS file and flash utility to a bootable floppy disk or USB drive.

2. Boot up the notebook form USB floppy disk drive or USB drive.

Make sure the AC adapter is firmly connected to your notebook before you run this utility.

4. Execute the batch file *.bat and press <Enter> key at DOS prompt A:\>.

5. Wait for a couple of seconds to complete the BIOS update procedure.

DO NOT power off the notebook during this BIOS update process.

6. Power off the notebook after the BIOS update procedure is completed.

7. Remove battery pack and AC adapter.

8. Plug in battery pack or AC adapter.

9. Power on the notebook and press <F2> key to enter the BIOS setup menu.

10. Select [Load Setup Defaults] and press <Enter> key to load BIOS default settings.

11. Select [Exit &Save Changes] and press <Enter> key to save changes and exit.